MSC IN ANALYSIS AND DESIGN OF EARTHQUAKE RESISTANT STRUCTURES (ADERS)

Course: Geotechnical Engineering in the Design of Structures

PROJECT: Diaphragm wall analysis

| Soil | Unit weight (KN/m3) | Young’s modulus E (MPa) | Friction Angle φ΄(degrees) | Cohesion intercept c΄ (kPa) | Permeabi-lity (m/s) |
| --- | --- | --- | --- | --- | --- |
| Fill | 21 | 10 | 33 | 0 | 10-4-10-6 |
| Formation A | 19 | 15 | 30 | 0 | 10-5 |
| Formation B | 20 | 30 | 30 | 5 | 5x10-7 |
| Formation C | 19 | 50 | 33 | 15 | 5x10-7 |

Table 1: Soil characteristics

| Excavation depth (m) | Dewatering depth (m) | Prop number | Depth (m) | Area (m2) | Moment of inertia (m4) |
| --- | --- | --- | --- | --- | --- |
| 4 | 5 | 1 | 3 | 218x10-4 | 6.6x10-4 |
| 10 | 11 | 2 | 9 | 354x10-4 | 36.0x10-4 |
| 14 | 14 | - | - | - | - |

Table 2: Excavation and water levels and prop levels and characteristics